|  | Service Bulletin EX59-24 Console Software Upgrade to S2.03 |  |
| --- | --- | --- |
| Effective Date: Jan.28.2022 | Models Affected: EX59-24（EP295C-2KM） | Bulletin Number NB-2201010 |
| Prepared by: Claire Nguyen |  |  |

Description:

This new version is launched to solve the below problems:

- When console is switched to English system (miles), the P2 program doesn’t show the correct distance at L9 and it ends at 20 but not 12.4.

- The initialization buzzer only rings twice.

Solution:

Upgrade EX59-24 console software from S1.03 to S2.03 with USB.

The upgrade software is available on CS Web or you can download it via the link below:

http://service.johnsonfitness.com/cs_new/software_sub.asp?id=337&brand=hz

Upgrade SOP:

- Prepare a USB drive, a computer and the upgrade software.

- Insert the USB drive into the computer and go to the root directory of the USB drive.

- Create a new file folder named HORIZON in the root directory of the USB drive.

- Create a new file folder named FWF in the file folder HORIZON.

- Rename the upgrade file (.bin) to EFM.bin.

- Put the upgrade file in the file folder FWF.

- Safely remove the USB drive from the computer.

- Remove the safety key from the console (or turn off the unit).

- Insert the USB drive into the console USB port.

- Reinsert the safety key (or turn on the unit).

- Follow the instruction on the console screen to finish the upgrade.

- Cycle the power, check if the console version is identical to the software version installed.

The solutions for the equipment in the field and warehouse

| 1 | Affected models (JHT no.) | EX59-24（EP295C-2KM） |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 | NA ECR no. | ECR22010065 | 10 | Old part number | 1000472706 |
| 3 | JHT ECO no. | ECO22010065 | 11 | old price | N/A |
| 4 | Implemented date | 24 January 2022 | 12 | New part number | 1000472706 (no change) |
| 5 | Started s/n | NA | 13 | new price | N/A |
| 6 | field service suggestion | □ case by case □ club by club ■ rework all | 14 | S-BOM updated | no |
| 7 | units in the warehouse | □ no action required ■ replace all | 15 | S-Drawing update | no |
| 8 | parts in stock | □ Use up ■ Rework □ Scrap | 16 | photos attached | no |
| 9 | video available | no | 17 | SOP attached | No |

|  | Bulletin type (Draft version or Final version) |
| --- | --- |
| □ | Draft version due to implement date and started S/N are not ready yet. |
| ■ | Final version, All completed |

Remarks:

Use up --- Use up the parts till empty
Rework -- Following parts rework SOP or update software

Scrap -----Contact with MC CS